PDC2021

Vienna, Austria

Please submit your abstract at https://atpi.eventsair.com/7th-iaa-planetary-defense-conference-2021/abstractsubmission

You may visit https://iaaspace.org/pdc

(please choose one box to be checked)

(you may also add a general comment - see end of the page)

Key International and Political Developments

Advancements and Progress in NEO Discovery

NEO Characterization Results

Deflection and Disruption Models & Testing

Mission & Campaign Designs

Impact Consequences

Disaster Response

Decision to Act

Public Education & Communication

Hijacking a satellite for Short-Warning Asteroid Deflection – FastKD Mission, Design and Implementation

Albert Falke(1), Karl Atkinson(1), Marc Chapuy(2)

(1)Airbus Defence and Space GmbH, 88090 Immenstaad, Germany, +49 7545-82535, albert.falke@airbus.com

(2)Airbus Defence and Space SAS, 31400 Toulouse, France, +33 56219 6320, marc.chapuy@airbus.com

Keywords: NEOs, Asteroid Kinetic Deflection, Short-warning, Emergency response, Impactor

##### ABSTRACT

The number of known Near-Earth Objects (NEOs) has increased exponentially in the recent years thanks to new observation programmes. Some of these objects have a very low but existing probability to impact Earth in near future. Therefore, global planetary defence activities are aiming the development of deflection techniques in order to prepare potential measures against such threats.

The emerging scenario addressed here assumes an asteroid, which has just been discovered and the orbit propagation analysis revealed a high probability of Earth impact within a very short warning time of approximately 1-3 years.

One (or multiple) kinetic asteroid deflection missions are identified as a viable deflection method, but because of the short warning time the consequence is an extremely constrained preparation time with a "to Launch" requirement of 6 months or less. This results in a build or system adaptation timeframe of 2-3 months.

The work described here has been completed in the ESA study called "Fast Development of Kinetic Impactor for Short-Warning Asteroid Deflection (FastKD)".

The study consequently assesses the feasibility of "hijacking" and modifying a commercial spacecraft platform in order to perform asteroid kinetic deflection in the shortest possible time. Moreover, the necessary prerequisites to enable the "build and launch of a Kinetic Impactor (KI) deflection system extremely fast" (Targeting for a 6 months launch readiness!) are identified. The system design approach follows the objective of minimal adaptations and the addition of a prepared Kinetic Deflection (KD) module in order to convert it to a deflection mission.

Finally a viable preliminary design solution is proposed compliant to several enveloping mission profiles, advanced GNC subsystem design iterations & performance demonstrations are accomplished and critical technology developments / long-lead items are identified within an implementation plan.

Acknowledgement, Disclaimer: This activity has been performed under a program of, and funded by, the European Space Agency. The view expressed by the authors in this publication does in no way reflect the official opinion of the European Space Agency.

**************************************

Comments:

Oral presentation very much preferred. Many thanks :)

Subtopic: Mission & Campaign Designs - NEO response mission campaign developments, including an international approach