PDC2021

Vienna, Austria

Please submit your abstract at https://atpi.eventsair.com/7th-iaa-planetary-defense-conference-2021/abstractsubmission

You may visit https://iaaspace.org/pdc

(please choose one box to be checked)

(you may also add a general comment - see end of the page)

Key International and Political Developments

Advancements and Progress in NEO Discovery

NEO Characterization Results

Deflection and Disruption Models & Testing

Mission & Campaign Designs

Impact Consequences

Disaster Response

Decision to Act

Public Education & Communication

Surface Gravimetry on Dimorphos with GRASS on Juventas

Birgit Ritter(1), Özgür Karatekin(1), Jose A. Carrasco(2), Matthias Noeker(1), Ertan Ümit(1), Emiel Van Ransbeek(3), Higinio Alaves(2), Elisa Tasev(1), Marta Goli(1), and Michel Van Ruymbeke(1)

(1)Royal Observatory of Belgium, Avenue Circulaire 3, 1180 Brussels, Belgium, birgit.ritter@observatory.be

(2)EMXYS, Elche/Elx, Spain

(3)Royal Belgian Institute for Space Aeronomy, Brussels, Belgium

Keywords: Gravimeter, surface acceleration, Dimorphos, Juventas/Hera

##### ABSTRACT

In the frame work of ESA’s Hera mission to the binary asteroid Didymos, the gravimeter for small solar system objects (GRASS) is being developed to measure the local acceleration vector on the surface of the system’s moonlet Dimorphos. GRASS will be onboard the Juventas CubeSat, which is one of the two daughtercraft of the Hera spacecraft. Launched in 2024, it will arrive in the binary system in 2026 to characterize the binary system and the aftermath of NASA’s Dart impact on the moonlet. Following the soft-landing of the Juventas CubeSat on Dimorphos, GRASS will record the temporal variation of the surface gravity vector at the landing location.

The average gravitational force expected on the Dimorphos surface is around 5 x 10-5 m s-2 (or 5 mGal). Apart from the self-gravitation of the body, centrifugal forces and the acceleration due to the main body of the system contribute to the surface acceleration. The temporal variations of local gravity vector at the landing site will be used to constrain the geological substructure (mass anomalies, local depth and lateral variations of regolith) as well as the surface geophysical environment (tides, dynamic sloped and centrifugal forces).

The gravimeter consists of two axes, each axis with a rotating flat spring, which are accommodated orthogonally in the Juventas CubeSat. This allows the reconstruction of the full three-dimensional local acceleration vector. The gravimeter will be calibrated in-situ.

We will present the GRASS instrument and science objectives within the Hera mission. Instrument development status as well as the operational concept will be described.

Acknowledgments: This project has received funding from BELSPO via the PRODEX Programme of ESA and from the European Union’s Horizon 2020 research and innovation program under grant agreement Nr. 870377 NEO-MAPP.

**************************************

Comments:

Session: Highlight/Spotlight Topics: Activities related to the two planetary defense missions in development: NASA DART and ESA Hera