PDC2021

Vienna, Austria

Please submit your abstract at https://atpi.eventsair.com/7th-iaa-planetary-defense-conference-2021/abstractsubmission

You may visit https://iaaspace.org/pdc

(please choose one box to be checked)

(you may also add a general comment - see end of the page)

Key International and Political Developments

Advancements and Progress in NEO Discovery

NEO Characterization Results

Deflection and Disruption Models & Testing

Mission & Campaign Designs

Impact Consequences

Disaster Response

Decision to Act

Public Education & Communication

DYNAMICS OF EJECTA PLUME AFTER THE DART IMPACT ON DIMORPHOS

Fabio Ferrari(1,2), Sabina D. Raducan(1), Martin Jutzi(1), Eugene G. Fahnestock(2), and Stephen R. Schwartz(3)

(1) Space Research and Planetary Sciences, University of Bern, Gesellschaftsstrasse 6, 3012 Bern, Switzerland, fabio.ferrari@space.unibe.ch

(2) Jet Propulsion Laboratory, California Institute of Technology, 4800 Oak Grove Dr, Pasadena, CA 91109, USA

(3) Lunar and Planetary Laboratory, University of Arizona, 1629 E. University Boulevard, Tucson, AZ 85721, USA

Keywords: DART, AIDA, high-velocity impact, ejecta, Didymos

##### ABSTRACT

The Double Asteroid Redirection Test (DART) is a NASA mission planned to be launched in 2021 and to perform a high-velocity impact on the surface of asteroid Dimorphos, the smaller component of Didymos binary system, in September/ October 2022. DART is part of the Asteroid Impact and Deflection Assessment (AIDA), alongside with ESA’s Hera mission, which is planned to visit Didymos in 2027.

We address the problem of dynamical evolution of the ejecta plume shortly after the DART impact on Dimorphos. The goal of this work is to assess the effects of the contact interactions between ejecta fragments, in the gravity regime, using N-body simulations.

We study the dynamics of ejecta fragments using GRAINS, a N-body Discrete Element Method (DEM) code, based on the multi-physics library Chrono::Engine. The translational and rotational dynamics of each ejecta fragment is propagated as a 6-DOF non-spherical rigid-body. Each fragment interacts with Didymos environment and other ejecta particles through gravity and contact. The gravitational environment considers self-gravity between ejecta particles, as well as the gravity of Dimorphos and Didymos (primary asteroid). Contact interactions are modelled using soft-contact (spring-dashpot system) and Coulomb friction model acting at contact points, which are identified at each time step between particles of non-spherical shape.

To model the properties of the ejecta plume moments after the DART impact, the DEM simulations are using initial conditions provided by ejecta scaling laws with parameters derived from shock physics numerical simulations. The information provided by the ejecta scaling laws include velocity and mass distribution within the ejecta plume. We study different dynamical scenarios, each represented by a different set of initial conditions – all related to the DART/Dimorphos scenario – and include a range in impacts angles, as well as different strength and initial porosities of the impacted surface of the target.

Preliminary results have shown that particle contact and particle shape have a very relevant effect on the short-medium evolution of the fragments. In particular, we observe non-spherical particles to have a lower global restitution coefficient due to energy dissipation of non-central collisions. In this case the translational energy is converted into spin energy of the single fragments. In this context, higher density structures are more likely to be formed within the ejecta cone. This might affect the long-term evolution of ejecta and their fate. Also, heterogeneous density regions within the ejecta cone might also affect the optical properties and observability of the plume shortly after DART’s impact.

**************************************

Comments:

Submission for oral presentation.

(From Call for Papers) Highlight/Spotlight topic area: Activities related to the two planetary defense missions in development: DART (NASA) and Hera (ESA).

This work is developed as part of the DART Ejecta Working Group.