PDC2021

Vienna, Austria

Please submit your abstract at https://atpi.eventsair.com/7th-iaa-planetary-defense-conference-2021/abstractsubmission

You may visit https://iaaspace.org/pdc

(please choose one box to be checked)

(you may also add a general comment - see end of the page)

Key International and Political Developments

Advancements and Progress in NEO Discovery

NEO Characterization Results

Deflection and Disruption Models & Testing

√ Mission & Campaign Designs

Impact Consequences

Disaster Response

Decision to Act

Public Education & Communication

Assembled Kinetic Impactor for Deflecting Asteroids via Combining the Spacecraft with the Launch Vehicle Upper Stage

Yirui Wang1, 2, Mingtao Li*1, 2, Zizheng Gong3, Jianming Wang4, Chuankui Wang4, Binghong Zhou1, 2

1National Space Science Center, Chinese Academy of Sciences, Beijing, 100190, China

2University of Chinese Academy of Sciences, Beijing, 101407, China

3Beijing Institute of Spacecraft Environment Engineering, China Academy of Space Technology, Beijing, 100094, China

4Beijing Institute of Astronautical Systems Engineering, China Academy of Launch Vehicle Technology, Beijing, 100076, China

*Correspondence Author E-mail: limingtao@nssc.ac.cn

Keywords: Kinetic Impactor, Asteroid Deflection, Launch vehicle upper stage, Near-Earth asteroid

##### ABSTRACT

Asteroid Impacts pose a major threat to all life on Earth. Deflecting the asteroid from the impact trajectory is an important way to mitigate the threat. A kinetic impactor remains to be the most feasible method to deflect the asteroid. However, due to the constraint of the launch capability, an impactor with the limited mass can only produce a very limited amount of velocity increment for the asteroid. To improve the deflection efficiency of the kinetic impactor strategy, this paper proposed a new concept called the Assembled Kinetic Impactor (AKI), which is combining the spacecraft with the launch vehicle upper stage. That is, after the launch vehicle upper stage sending the spacecraft into the Earth escaping trajectory, the spacecraft-rocket separation will not be performed and the spacecraft controls the AKI to impact the asteroid. By making full use of the mass of the launch vehicle upper stage, the mass of the impactor will be increased, which will cause the improvement of the deflection efficiency. According to the technical data of the Long March 5 (CZ-5) launch vehicle, the missions of deflecting Bennu are designed to demonstrate the power of the AKI concept. Simulation results show that, compared with the Classic Kinetic Impactor (CKI, performs spacecraft-rocket separation), the addition of the mass of the upper stage can increase the deflection distance to more than 3 times, and reduce the launch lead-time. With the requirement of the same deflection distance, the addition of the mass of the upper stage can reduce the number of launches to 1/3 of that of the number of CKI launches. The AKI concept makes it possible to defend Bennu-like large asteroids by a no-nuclear technique within a 10-year launch lead-time. At the same time, for a single CZ-5, the deflection distance of a 140 m diameter asteroid within a 10-year launch lead-time, can be increased from less than 1 Earth radii to more than 1 Earth radii, which means the improvement of the reliability and efficiency of asteroid deflection missions.