PDC2021

Vienna, Austria

Please submit your abstract at https://atpi.eventsair.com/7th-iaa-planetary-defense-conference-2021/abstractsubmission

You may visit https://iaaspace.org/pdc

(please choose one box to be checked)

(you may also add a general comment - see end of the page)

Key International and Political Developments

Advancements and Progress in NEO Discovery

NEO Characterization Results

Deflection and Disruption Models & Testing

Mission & Campaign Designs

Impact Consequences

Disaster Response

Decision to Act

Public Education & Communication

JuRa: the Juventas Radar on Hera to fathom Didymoon

Alain Herique(1), Dirk Plettemeier(2) ,Wlodek Kofman(1), Hannah Goldberg(3), Yves Rogez(1), Lucas Cicero(4) and JuRa Team

(1) Univ. Grenoble Alpes, CNRS, CNES, IPAG, F-38000 Grenoble, France,

alain.herique@univ-grenoble-alpes.fr

(2) Technical University Dresden, 01187 Dresden, Germany

(3) GomSpace, Aalborg East, Denmark

(4) EmTroniX, Hautcharage, Luxembourg

Keywords: Radar, Sounding, Asteroid, internal structure, HERA mission

##### ABSTRACT

The ESA Hera mission approved by the last ESA council Space19+, will be launched in 2024 to deeply investigate the Didymos binary system. In 2022, DART/NASA will impact the moonlet to quantify the mechanical response of the body, mainly from ground-based observation. Five years later, HERA/ESA is a unique opportunity to observe in detail the bodies, the crater and the ejecta in order to better constrain the mechanical models. Hera will provide a global characterization of the binary system: shape, density, dynamic properties, thermal properties and composition.

Hera will release a CubeSat, named Juventas, once in proximity to the Didymos system.  Onboard Juventas, the JuRa radar will fathom Didymoon and provide the first direct observation of an asteroid deep interior. The JuRa instrument is a monostatic synthetic aperture radar, BPSK coded at 60MHz carrier frequency and 20MHz bandwidth.

The first objective of JuRa is to characterize the moonlet’s interior, to identify internal geological structure such as layers, voids, sub-aggregate, to bring out the aggregate structure and to characterize its constituent blocks in terms of size distribution and heterogeneity at different scales (from submetric to global).

The second objective is to estimate the average permittivity and to monitor its spatial variation in order to retrieve information on its composition and porosity, especially in the area of the impact crater. Radar bypasses the near surface alteration by space-weathering and thermal-cycling as observed with optical remote sensing.

The same characterization applied to the main asteroid of the binary system is among the secondary objectives, to detect differences in texture and composition and to support the modeling of the binary system’s formation.  Supporting shape modeling and the determination of the dynamic state through radar ranging are other secondary objectives.

Thus, the observation of the structure and composition of moonlet will provide constraints on the mechanical model of the impact process. When compared to the observation of the main body, it will constrain the model of binary system formation to discriminate between progressive versus catastrophic process and more generally on the stability condition of the system.

Acknowledgement:

Hera is the ESA contribution to the AIDA collaboration.

Juventas and JuRa are developed under ESA contract supported by national agencies. JuRa is built by EmTroniX (Lux), IPAG UGA/CNRS (Fr), TUD (Gr), Astronika (Pl) and BUT (Cz). Juventas is built by GomSpace (Lux).

The authors acknowledge funding support from the European Union’s Horizon 2020 research and innovation program under grant agreement No 870377 (project NEO-MAPP).

**************************************

Comments:

Oral presentation preferred.

The presenter works in Central European Time. Please take this into account when planning the presentation times.