PDC2021

Vienna, Austria

Please submit your abstract at https://atpi.eventsair.com/7th-iaa-planetary-defense-conference-2021/abstractsubmission

You may visit https://iaaspace.org/pdc

(please choose one box to be checked)

(you may also add a general comment - see end of the page)

Key International and Political Developments

Advancements and Progress in NEO Discovery

NEO Characterization Results

Deflection and Disruption Models & Testing

Mission & Campaign Designs

Impact Consequences

Disaster Response

Decision to Act

Public Education & Communication

Planetary Defense Ground Zero: MASCOT’s View on the Rocks – an Update between First Images and Sample Return

Tra-Mi Ho(1), Ralf Jaumann(2), Jean-Pierre Bibring(3), Matthias Grott(4), Karl-Heinz Glaßmeier(5), Aurelie Moussi(6), Christian Krause(7), Ulrich Auster(5), Volodymyr Baturkin(1), Jens Biele(7), Federico Cordero(8), Barbara Cozzoni(7), Clement Dudal(6), Cinzia Fantinati(7), Christian D. Grimm(1), Jan Thimo Grundmann(1)*, Maximilian Hamm(4,9), Jeffrey Hendrikse(14), David Herčík(10), Kağan Kayal(7), Jörg Knollenberg(4), Oliver Küchemann(7), Caroline Lange(1), Michael Lange(11), Laurence Lorda(6), Michael Maibaum(7), Yuya Mimasu(12), Céline Cenac-Morthe(6), Tatsuaki Okada(12), Katharina Otto(4), Cedric Pilorget(3), Josef Reill(13), Takanao Saiki(12), Kaname Sasaki(1), Markus Schlotterer(1), Nicole Schmitz(4), Stefan Schröder(4), Nawarat Termtanasombat(14), Nortbert Tóth(1), Yuichi Tsuda(12), Stephan Ulamec(7), Friederike Wolff(4), Tetsuo Yoshimitsu(12), Christan Ziach(14), and the MASCOT Team

(1)DLR German Aerospace Center, Institute of Space Systems, Robert-Hooke-Strasse 7, 28359 Bremen, Germany – Tra-Mi.Ho@dlr.de

*corresponding coauthor +49-(0)421-24420-1107, jan.grundmann@dlr.de

(2)Freie Universität Berlin, Institute of Geological Sciences, Berlin, Germany

(3)Institute d’Astrophysique Spatiale, Orsay, France

(4)DLR, Institute of Planetary Research, Berlin, Germany

(5)IGEP, Technische Universität Braunschweig, Braunschweig, Germany

(6)Centre National d'Etudes Spatiales (CNES), Toulouse, France

(7)DLR, Institute of Space Operations, Cologne, Germany

(8)Telespazio VEGA, Darmstadt, Germany

(9)Institute for Mathematics, University of Potsdam, Karl-Liebknecht-Str. 24-25, Potsdam-Golm, Germany

(10)Institute of Atmospheric Physics, Czech Academy of Sciences, Czech Republic

(11)DLR, Institute of Composite Structures and Adaptive Systems, Braunschweig, Germany

(12)JAXA, Institute of Space and Astronautical Science, Sagamihara, Japan

(13)DLR, Institute of Robotics and Mechatronics, Oberpfaffenhofen, Germany

(14)consultants to DLR, Institute of Space Systems, Bremen, Germany

Keywords: asteroid surface properties, planetary defense, MASCOT, (162173) Ryugu, Hayabusa2

##### ABSTRACT

At 01:57:20 UTC on October 3rd, 2018, after 3½ years of cruise aboard the JAXA spacecraft Hayabusa2 and about 3 months in the vicinity of its target, the MASCOT lander was separated successfully by from an altitude of 41 m. After a free-fall of only ≈5m51s MASCOT made first contact with C-type near-Earth and potentially hazardous asteroid (162173) Ryugu, by hitting a big boulder. MASCOT then bounced for ≈11m3s, in the process already gathering valuable information on mechanical properties of the surface before it came to rest. It was able to perform science measurements at 3 different locations on the surface of Ryugu and took many images of its spectacular pitch-black landscape. MASCOT’s payload suite was designed to investigate the fine-scale structure, multispectral reflectance, thermal characteristics and magnetic properties of the surface. Somewhat unexpectedly, MASCOT encountered very rugged terrain littered with large surface boulders. Observing in-situ, it confirmed the absence of fine particles and dust as already implied by the remote sensing instruments aboard the Hayabusa2 spacecraft. After some 17h of operations, MASCOT‘s mission ended with the last communication contact as it followed Ryugu’s rotation beyond the horizon as seen from Hayabusa2. Soon after, its primary battery was depleted. We present a broad overview of the recent scientific results of the MASCOT mission from separation through descent, landing and in-situ investigations on Ryugu until the end of its operation and relate them to the needs of planetary defense interactions with asteroids. We also recall the agile, responsive and sometimes serendipitous creation of MASCOT, the two-year rush of building and delivering it to JAXA’s Hayabusa2 spacecraft in time for launch, and the four years of in-flight operations and on-ground testing to make the most of the brief on-surface mission.

**************************************

Comments: oral presentation preferred, poster (or equivalent) welcome

(Alternative session, Time slot, Oral or Poster, Etc…)